SEMI-AUTOMATIC STEREOLOGY MACRO STEPS (10.10.2019)

This macro allows the user to implement a semi-automatic stereology of the liver of a stack of T2-dicom images with the software ImageJ. In order to make the macro work, the plugin “Grid” (it’s a modified version – attached at the end of the description of the procedure) must be installed in the ImageJ’s “Plugins” folder.

- Open ImageJ;

- Run the plugin: ImageJ → Plugins → Macro → Run → Semi-Automatic stereology macro;

- Follow the instruction of the macro, press “OK” on the “Action Required” window and select the directory that contains the folder of T2 DICOM sequence images.

- The following window will pop up:

In the “Stack ROIs” window, click “Read ROIs” and select the ROI of the liver (previously realized):

The ROI previously realized will automatically line up the sequence of images. The overlap may not be perfect, since:

  - the number of images of the T2-Stack is different from the stack used for the manual tracking (before launching the macro it is recommended to delete those images of the sequence which are empty);

  - if the ROI is not precise, move it with the cursor in order to improve the overlap;

After that, press “OK” on the previous “Action Required” window.

- Follow the instruction of the macro, close the ROI Manager and press the button Save as Overlay. After that, press the button “OK” on the “Action Required” window.

- The end of the entire process is reported by the following window:

- At the end of the process the ROI Manager is shown in the following image:

  - Show all: show all points on the corresponding images of the sequence;

  - Labels: assign to every point an increasing number;

  - ROI Manager → More → List; a list of the overlay elements of the sequence is opened. It contains all points detected (the number of all points correspond to the last point “index” value +1, since the numbering starts at 0);

a) in the ROI Manager are loaded all points automatically imported; the points are not shown on the image

b) if “Show all” is checked, points are shown on the image

- Manual correction:

  - to delete any wrong point, you can select the point directly on the image; in the ROIManager, select “Delete” in order to remove it permanently;

  - to add any point, you can select the corresponding cross with the “Multi-Point” tool of ImageJ and, by clicking “t”, it will be automatically added to the ROIManager list;

To save the points: ROIManager → Deselect → More → Save → “RoiSet.zip” → select the directory;

The automatically-detected points are now saved. To visualize them, open the Red-Grid Image sequence and load the RoiSet.zip compressed file as follows:  Analyze → Tools → ROIManager → More → Open → select the “RoiSet.zip”